N-BOX-J7（R100）

Product features:

- Onboard Intel Alder Lake-N serial processor

- 1*DDR5 SO-DIMM，Max 16GB

- 4*2.5G network ports,choose POE

- TPM2.0，1*M.2 2280 SSD，MiniPCIe(WIFI/BT/4G/mSATA)

- HDMI2.0+DP，1*Type-C（USB+DP）

- DC 12V，36W；Size：134x126x41mm

Product specification

| Processor | - Onboard Intel Alder Lake-N serial processor,TDP 15W |
| --- | --- |
|  | - EFI BIOS |
| Ram | - 1*DDR5 SO-DIMM，Max 16GB |
| Storage | - 1*M.2 M-Key 2280(NVMe PCIe 4.0 x2/SATA3.0 protocol) |
|  | - 1*SATA3.0 slot，2Pin 5V power |
| Display port | - 1*HDMI2.0,support 4096x2160@60Hz |
|  | - 1*DP,support 7680x4320@60Hz |
|  | - Type-C,support DP 7680x4320@60Hz and USB3.2 Gen2 |
| I/O interface | - Power on button, reset button, 2*USB3.0, 2*USB2.0, TF card |
|  | - DC JACK, HDMI+DP, Type-C |
|  | - 4*2.5G LAN (3*Intel 226-V, 1*RTL8156B, optional POE), RJ45 console |
| Extended Interface/Function | - External TPM2.0 is optional, not available by default. Default is CPU built-in TPM2.0. |
|  | - 1*MiniPCIe (PCIe+USB2.0+SATA3.0 protocol, WIFI/BT/mSATA/4G module),Micro SIM card |
|  | - 5*RS232/RS485 pin (hardware material selection), pitch 2.0mm; 8*GPIO |
|  | - 1*4Pin PWM CPU FAN; 1*4Pin PWM SYS FAN |
| Power supply | - DC 12V，36W Above |
| working environment | - Operating Temperature: -20℃ ~ +60℃; Operating Humidity: 5% ~ 90 |
|  | - Storage Temperature: -40℃ ~ +85℃; Storage Humidity: 5% ~ 90% |
| operating system support | - Windows10，Windows11，Linux |
| Size | - 134x126x41 mm |
| Net weight | - About 600g |

N-BOX-J7（R100）

Product size                                           unit：mm

Ordering information

| Model | Optional | Optional description |
| --- | --- | --- |
| - N-BOX-J7 | - Onboard Intel Alder Lake-N Series Processor | - N95/N100/N305 |
|  | - POE | - POE optional |
|  | - Type-C | - Note required for order |
|  | - Serial port pin optional (CONSOLE always has one) | - Note required for order |
|  | - RS232/RS485 Optional | - RS232/RS485 selectable by hardware material |
|  | - TPM2.0 | - External TPM2.0 optional |