N-BOX-T5

◆ Integrated Intel Gemini Lake single CPU；

◆ Support TPM2.0(optional)

◆Provide 1*VGA,1*HDMI 1.4（Support 4K display output）；

◆COM1,2 can be selected as RS232 or RS485 by jumper and BIOS settings, COM3, COM4, COM5, COM6 are RS232

◆ onboard 32/64/128G EMMC storge chipset（optional）;

◆ Support power-on automatic boot function, jump selection；

◆ Wide viltage 12-19V power supply；

◆ Size 134*126*46.8 mm

Specification

| Processor system | CPU | Integrated Intel Gemini Lake single CPU |
| --- | --- | --- |
|  | CPU package | BGA |
|  | Chipset | Intel Gemini Lake serial SOC |
|  | BIOS | EFI BIOS |
| Memory | Technology framework | Single channel DDR4 2133Mhz |
|  | Memory | MAX Support DDR4 8G |
|  | Socket | DDR4 SODIMM 60 Socket |
| Video | Graphic controller | Intel UHD 600 Graphics |
|  | HDMI | HDMI（2.0）MAX resolution 4K@60Hz |
|  | VGA | VGA MAX resolution 1920x1200 |
| I/O rear panel | ports | 1*HDMI 2*USB3.0 4*USB2.0 2*LAN 6*COM(COM1,2 can be selected as RS232 or RS485 by jumper and BIOS settings，RS485 supports automatic flow control，COM3,COM4,COM5,COM6 are RS232) 1*Quick boot button with indicator light 1*Reset button 1*DC 1*Power indicator light 1*Hard disk indicator light |
| Network | Controller | 2*Intel I211AT or Intel I210High-performance gigabit network card(Optional) |
| Input/output interface | USB | 4*USB2.0 2*USB3.0 Max support +5V/1A |
|  | Com port | COM1,2 can be selected as RS232 or RS485 by jumper and BIOS settings，RS485 supports automatic flow control，COM3,COM4,COM5,COM6 are RS232 |
| Expansion bus | Mini-PCle | 1 Mini-PCIe socket,support PCIe and USB device（5G module or 4G module optional） |
| Storge | SATA | 1 standard SATAIII interface with a maximum transfer rate of 6Gb/s |
|  | M-SATA | 1*Mini-PCIe M-SATA Scoket,Support SANDISK protocol, maximum transfer rate of 6gb/s |
|  | EMMC | Support 32/64/128G(optional） |
| Power supply | Power Type | Single input dc power supply,DC12-19V |
| Working environment | Operating temperature | -20℃ ~ +60℃ |
|  | Storage temperature | -40℃ ~ +85℃ |
|  | Working humidity | 0% ~ 90% relative humidity, no condensation |
|  | Storage humidity | 0% ~ 90% relative humidity, no condensation |
| System Support |  | Windows10 Linux |
| Physical dimension | Size | 134*126*46.8 mm |
|  | Weight | 0.5KG |
| Certification |  | CE,ROHS,FCC |

N-BOX-T5

Size

单位：mm

Order Info

| Item NO. | Spec | CPU optional |
| --- | --- | --- |
| GN41NA | Integrated Intel Gemini Lake single chip CPU | J4105/J4005/N4100 |